DeviceLab

Infrastructure Sizing for 20 Android Devices

On-prem remote mobile device lab · Hardware, network and capacity requirements

| Item | Detail |
| --- | --- |
| Scope | 20 Android devices streaming at the same time, with room to grow to about 40 |
| Deployment | On-prem only, single control server plus Android host machines |
| Basis | Derived from the DeviceLab code, stream bit rates and ADB/USB limits. Not load-tested |
| Date | 15 October 2026 |

# 1. Summary

A 20-device lab needs one control server and two Android host machines with ten phones each. The control server relays video and stores builds; the hosts drive the phones over USB. Video is never re-encoded, so CPU needs are modest. The real limits are USB controller capacity, hub power and heat.

| Component | Qty | Recommended spec |
| --- | --- | --- |
| Control server (VM or physical) | 1 | 8 vCPU, 16 GB RAM, 500 GB SSD, 1 GbE |
| Android host PC | 2 | 4–6 cores, 16 GB RAM, 256 GB SSD, multi-controller USB 3, 1 GbE |
| Powered USB 3 hub (10-port, ≥ 2.4 A per port) | 2–3 | Industrial grade, individually powered ports |
| USB cables (data-rated, 0.5–1 m) | 20 + spares | Short, good-quality cables |
| Network switch ports and UPS | — | 1 GbE port for each host and the server; UPS for server and hosts |

These figures are engineering estimates, not load-test results. Run a pilot with your own phones before buying hardware.

# 2. Sizing basis

## 2.1 How the system uses resources

- Video is passed through. The phone encodes H.264 itself; the host agent and server only forward the bytes. Video therefore costs very little CPU.

- Bandwidth follows the quality preset. Each live device streams at a fixed bit rate (table below).

- USB and power are the usual limits. Running out of USB controller capacity, weak hubs and heat cause more problems than processing power.

## 2.2 Stream quality presets

| Preset | Resolution | Frame rate | Bit rate per device | 20 devices |
| --- | --- | --- | --- | --- |
| Low | 800 px | 24 fps | 1.5 Mbit/s | 30 Mbit/s |
| Medium (default) | 1280 px | 30 fps | 4 Mbit/s | 80 Mbit/s |
| High | 1920 px | 60 fps | 8 Mbit/s | 160 Mbit/s |

Each extra person watching a session adds one more stream at the same bit rate on the server-to-browser side.

# 3. Control server

One VM or physical server runs every central component in Docker Compose: Nginx (TLS), the NestJS application server, Postgres 16, Redis 7 and MinIO (or plain disk storage).

## 3.1 Specification

| Resource | Minimum (20 devices) | Recommended (20–40 devices) |
| --- | --- | --- |
| CPU | 4 vCPU | 8 vCPU |
| RAM | 8 GB | 16 GB |
| Disk | 200 GB SSD | 500 GB SSD (NVMe preferred) |
| Network | 1 GbE | 1 GbE (10 GbE only beyond about 100 devices) |
| Operating system | Ubuntu 22.04/24.04 or RHEL 9 with Docker | Same |

## 3.2 Memory by component

| Component | Approximate RAM |
| --- | --- |
| Nginx | Under 100 MB |
| DeviceLab server (NestJS) | 0.5–1 GB |
| Postgres 16 | 1–2 GB |
| Redis 7 | Under 200 MB |
| MinIO (object storage) | 0.5–1 GB |
| Operating system and headroom | 2–4 GB |

## 3.3 Network load with 20 live sessions

| Preset | In from host agents | Out to browsers |
| --- | --- | --- |
| Medium | About 80 Mbit/s | About 80 Mbit/s + 4 Mbit/s per extra viewer |
| High | About 160 Mbit/s | About 160 Mbit/s + 8 Mbit/s per extra viewer |

Even the High case is well below the capacity of a 1 GbE link.

## 3.4 Storage

| Data | Estimate | Notes |
| --- | --- | --- |
| App builds (APK) | 20–50 GB | For example 10 projects × last 20 builds × 50–200 MB each |
| Postgres, including audit log | Under 5 GB per year | Reservations, sessions, users, audit trail |
| Screenshots and logs | A few GB | Suggested 30-day retention |
| Session recordings (Phase 3, not built yet) | About 1.8 GB per hour per recording | Size the disk for this if you plan to record a lot |

# 4. Android host machines

Host machines run the DeviceLab Host Agent and have the phones plugged in over USB.

## 4.1 Why two hosts with ten phones each

- USB controllers run out of capacity. Many PC USB controllers start dropping devices at roughly 15–20 ADB devices on one controller, even when spread across several hubs.

- Fault isolation. One host failing takes down only half of the lab.

## 4.2 Specification per host (10 devices)

| Resource | Specification |
| --- | --- |
| CPU | 4 cores minimum (Intel i5 / Ryzen 5 class); 6–8 cores if you also install builds and stream on many devices at once |
| RAM | 8 GB minimum, 16 GB recommended |
| Disk | 128–256 GB SSD (cache for builds before they are installed) |
| USB | USB 3 ports on separate controllers, or a PCIe USB 3 card with several independent controllers |
| Hubs | Powered, industrial-grade hubs with at least 2.4 A per port (e.g. 2 × 7-port or 1 × 10-port per host) |
| Network | Wired 1 GbE to the control server (Wi-Fi not recommended) |
| Operating system | Ubuntu 22.04/24.04 recommended; Windows works but USB drivers are less predictable |

## 4.3 Expected load per host (10 live streams)

| Metric | Expected |
| --- | --- |
| CPU | About 10–25% (mostly adb and Node relaying data) |
| RAM | About 1–2 GB used |
| Network out | About 40–80 Mbit/s |

## 4.4 Single-host alternative (all 20 phones)

If only one host is possible: 8 cores, 16 GB RAM, and at least 2–3 independent USB controllers (a multi-controller PCIe card). Put no more than about 10 phones on each controller. The whole lab goes offline if this host fails.

# 5. Network requirements

| Path | Protocol and port | Notes |
| --- | --- | --- |
| Browser → Nginx (server) | HTTPS and WSS, TCP 443 | The only port users reach |
| Host agent → server | WSS, TCP 443 | Agents use per-host revocable tokens |
| Postgres, Redis, MinIO | Internal Docker network only | Never exposed outside the server |

- Keep user-to-server latency under about 30 ms so remote touch feels responsive. Same-city LAN or WAN is fine.

- Each user needs about 4 Mbit/s per open session at Medium quality.

- Users need a browser with H.264 decoding: current Chrome or Edge.

- Put the device lab on its own network segment (VLAN) that reaches only the DeviceLab server.

# 6. Physical setup

- Rack and cooling. 20 phones charging and encoding video produce a noticeable amount of heat. Keep airflow going or add a fan tray.

- Power. Use a UPS for the server and hosts, and keep hubs on surge-protected power.

- Phones. Keep "Stay awake while charging" on. Where supported, cap charging at about 80% to stop batteries swelling, since the phones are on charge all the time.

- Security. Keep the rack in a locked room and reconcile the device inventory monthly (see SECURITY.md).

# 7. Optional extras

| Item | Recommendation | Status |
| --- | --- | --- |
| iOS devices | macOS host, e.g. Mac mini M2/M4 with 16 GB RAM for about 5–8 iPhones | Phase 2 — iOS support is a placeholder in the code today |
| High availability | Second server with Postgres replication | Optional; a single instance is enough for 20 devices |
| Backups | Nightly pg_dump plus a copy of the build store on separate storage; test restores quarterly | Recommended |

# 8. Next steps

- Run a pilot with 5–10 of your own phones on one host and measure CPU, RAM, USB stability and bandwidth.

- Confirm the USB controller layout of the chosen host hardware before buying hubs.

- Decide on recording usage (Phase 3) and size the server disk accordingly.

- Agree network segmentation and TLS certificates with the security team.